| المملكة العربية السعودية جامعة الإمام محمد بن سعود الإسلامية كلية علوم الحاسب والمعلومات |  | Kingdom Of Saudi Arabia Al-Imam University faculty of Computer Science and Information |
| --- | --- | --- |

Midterm 1 exam

| Course Title: Computer Networks | Course Number: CS331 |
| --- | --- |
| Instructor(s): Dr. Miled Tezeghdanti & Dr. Rafe Alasem | Section(s): 171, 172, 173 |
| Duration: 1h15 | Date: 06-01-1431 |
| Number of Questions: 4 | Number of pages: 5 |
| Additional Materials: None | Type: Closed Book |

| Exercise 1 (5) | Exercise 2 (5) | Exercise 3 (4) | Exercise 4 (6) | Total (20) |
| --- | --- | --- | --- | --- |

|  | اسم الطـالب |
| --- | --- |
|  | الرقم الجامعي |
|  | الشعبة |
|  | المدرس |

Exercise 1: (5 points)

## Match between the suitable word(s) and the correct  answer in the following table:

| 8 | propagation | 1 | With __________ there is a dedicated path through the network between two end stations |
| --- | --- | --- | --- |
| 5/2 | packet | 2 | In ________ networks, the routing decisions that are made change as conditions on the network change. |
| 4/2 | datagram | 3 | Communication via circuit switching involves three phases: data transfer, circuit disconnect and __________. |
| 1 | Circuit switching | 4 | It is up to the exit node or the destination to detect the loss of a packet and decide how to recover it. In this technique, each packet, treated independently, is referred to as a __________. |
| 9 | processing | 5 | A network layer protocol data unit is often referred to as a ___________ . |
| 2/4 | Packet Switching | 6 | The protocol for the exchange of Web-based information between Web browsers and Web servers is __________ . |
| 3 | circuit establishment | 7 | The ________ is a directory lookup service that provides a mapping between the name of a host on the Internet and its numerical address. |
| 10 | error | 8 | The time for one bit to transit from source to destination is called ________ delay. |
| 6 | HTTP | 9 | The time required to process the packet at the source prior to sending, at any intermediate router or switch, and at the destination prior to delivering to an application is __________ delay. |
| 7 | Domain Name System (DNS) | 10 | Flow control, congestion control and _________ control are all difficulties for the protocol designer. |

Exercise 2: (5 points)

Write 'T' if the statement is true and 'F' if the statement is false.

| - F | - 1) A transport protocol can be either connection oriented, such as UDP, or connectionless, such as TCP. |
| --- | --- |
| - T | - 2) The complexity of a reliable connection-oriented transport protocol is caused by the need to deal with relatively large, variable delays between end systems. |
| - T | - 3) The transport protocol provides services to transport service users such as FTP, SMTP, and TELNET. |
| - T | - 4) With the stop-and-wait technique only one PDU at a time can be in transit between source and destination. . |
| - F | - 5) A TCP entity within a host cannot simultaneously provide service to multiple processes. |
| - F | - 6) TCP uses a sliding-window flow control mechanism that allows only one segment to be in transit at a time. |
| - F | - 7) When a TCP entity detects the presence of congestion in the Internet, it increases the flow of data onto the Internet until it detects a decrease in congestion. |
| - T | - 8) FTP uses TCP in order to make the connection necessary to send files. |
| - T | - 9) In the datagram approach, each packet is treated independently, with no reference to packets that have gone before. |
| - T | - 10) The SMTP protocol is used to transfer a message from a SMTP sender to a SMTP receiver over a TCP connection. |

Exercise 3: (4 points)

Suppose that we want to send 200,000 bit file over a circuit-switched network over a link of 1Mbps with 10 slots. What will be the total time that is needed to transfer this file?

Assume:

  - 10ms one-way latency

  - 50 msec to establish end-to-end circuit

(available rate) = 1Mbps/10=100Kbps=100 000 bps

Trans time = 200 000/ 100 000=2 s

Total time = Trans time+one-way delay+ end-2-end est= 2+0.01+0.05=2.06 s =2060 ms

-----------------------------------------------------------------------------------------------------------------------

Exercise 4: (6 points)

For the next two questions, consider the following network topology, with hosts A and B connected through router R:

Router R operates in store-and-forward mode. Links 1 and 2 are both 1M bit per second links with 10ms one-way latency.

- How long does it take to send a 1000 bit packet from A to B?

Trans time= 2L/R=2*(1000/1000 000)=2ms

Total time = 2ms+2*10=22ms

- Using stop-and-wait flow control, how long does it take for A to send a 100,000 bit file to B? Assume that ACKs are 100 bits long and that the data packets are 1000 bits long.

Total time ACK= 2*10+2(100/1000 000)=20.2ms

Total time _file= 100(20.2+22)=4220 ms = 4.22 s

- Assume the router has a buffer of 5000 bits, what is the maximum queuing delay that a packet may stay in the router buffer?

Max queuing delay = 5000/1000 000=5 ms